The Standard Quality Assurance Requirements is an integral part of the purchase order when it is invoked.  The paragraphs listed below are numbered to correspond with the numbers listed on the purchase order.

# QUALITY SYSTEMS

- The supplier shall maintain a Quality System in compliance to the requirements of ISO 9001, AS9100, ISO 17025 & AS9120 for distributors.

  - Quality records created by an /or retained by the supplier shall be available for review by Komtek Forge, its customers and/or Regulatory Authorities upon request. Unless otherwise specified on the purchase order the supplier shall maintain quality records for (30) years after the last delivery of products and/or services on the contract. The buyer must also be notified in writing prior to destruction of records.

  - Outside service suppliers that perform special processes (i.e. materials testing, NDT, welding, heat treat, etc.) must hold a NADCAP certification for the process being performed.

- The suppler shall maintain an identification and verification program to provide traceability of material while under their control.

  - All master heats and bar sizes must be segregated by individual bundles. Small individual bundles can be bundled together as long as each bundle is properly identified during shipment.

- DELETE

- Supplier shall pass on to any sub-tier supplier the applicable requirements of this purchase order.

# MATERIAL CERTIFICATION/ INSPECTION RECORDS

- Supplier shall supply a Certified Material Test Report providing mill identity, mill tests and analyses of material including the chemical and physical properties per the applicable specification.

  - DELETE

- Alloy suppliers shall provide certification as to the country of origin of the material provided. In addition, distributors are required to adhere to specialty metals DFARS clause requirements when specified on the Buyer Purchase Order.

- Supplier shall verify the material provided does not contain minerals tin, tantalum, tungsten or gold sourced from the Democratic Republic of the Congo or its adjoining countries. These have been identified as conflict minerals pursuant to Section 1502 of the Dodd-Frank Wall Street Reform and Consumer Protection Act.

- Alloy bar must meet a 63 surface finish

- Supplier shall submit a certification that includes cure or manufacturing date for material whose acceptability is limited by age. The shelf life shall be listed on the container and on the document certifying the material.

- Supplier shall certify that all articles furnished against this order does not contain or has been in contact with metallic mercury or mercury compounds.

- Supplier’s certification shall list the specification and revision level to which material is supplied.

# DEVIATIONS/NONCONFORMANCE

- Any departure from drawings, specifications or other customer requirements must be promptly reported to Komtek Forge. Disposition of these nonconformance’s must be approved in writing be Komtek Forge or its customer.

# SPECIAL PROCESSES

- Supplier shall submit a written description/ technique sheet of any Special Processes used.

# CERTIFICATION OF CONFORMANCE

- Supplier shall submit with each shipment a Certificate of Conformance (C of C) This certificate must reflect the revision level of the drawing and/or the specifications referenced by the order. A responsible representative of the supplier must sign the C of C.

- By making shipment under this purchase order the supplier automatically certifies that materials used in the product shipped and the processes applied comply with the applicable drawing and/or specifications. Supplier agrees to retain objective evidence and/or records to substantiate significant activities in the manufacturing, inspection, testing, treating, preservation and packing of delivered product. Such records shall be made available on request.

# MANUFACTURING INSPECTION AND TEST

- Komtek Forge must be notified prior to changes in product and/or process, changes of suppliers, changes of manufacturing facility location and, where required, obtain organizational approval.

- Process and procedure sheets are required for the following end use customers specifications;

  - GE Specification S-1000

  - PWA END USE specification ASQR-01

  - PWC END USE specification ASQR-01

  - Sikorsky END USE

# SPECIAL REQUIREMENTS

    - This order contains “Key Characteristics” which require 100% inspection prior to shipment.

# PACKAGING/PROTECTION

- Forgings shall be protected by appropriate material to prevent damage during shipment.

  - Raw materials purchased by KT shall be packaged appropriately to prevent damage during shipment, KT will not receive for payment any materials damaged during shipment. If needed, KT will return damaged materials to vendor for review at the vendor's expense

  - KT is not responsible for the return shipment of vendor packaging materials; KT will however return packaging materials upon request at the vendor's expense. KT will hold packaging materials for 5 business days after which packaging materials will be discarded if no such request for return shipment is received.

- Supplier shall package items for shipment so to prevent metal-to-metal contact on machined surfaces.

- When protective devices accompany product when shipped to supplier, the supplier shall maintain this protection during processing and return with the same protective device.

- Maximum length of alloy bars is 15’.

# TRAINING

21) The personnel performing work affecting quality are competent on the basis of appropriate education, training, skills and experience.

| Rev | Description | Date |
| --- | --- | --- |
| 0 | New Release | 10/10/07 |
| 1 | Paragraph 15c changed ES6-26 to PD1000 and revision (Any conflicts PD1000 takes precedence). Added paragraph 20 Maximum 15’ bars. Paragraph 1a changed from 10 to 15 years. Added TRAINING paragraph 21 | 9/3/08 |
| 2 | Paragraph 1a added; The buyer must also be notified in writing prior to destruction of records. | 2/20/09 |
| 3 | Paragraph 15c changed PD1000 Rev. Y to PD1000 Rev. AA | 3/20/09 |
| 4 | Added paragraph 2a, Paragraph 15C changed Rev AA to AB. Added PWA, PWC, SAC end use. Added paragraph 5a. Paragraph 1a changed from 15 to 30 years | 6/21/10 |
| 5 | Added paragraph 17a & 17b | 5/21/12 |
| 6 | Added paragraph 6a | 12/4/13 |
| 7 | Added paragraph 6b | 5/18/15 |
| 8 | - Changed paragraph 14 to: KomTek Technologies must be notified of changes in product and/or process, changes of suppliers, changes of manufacturing facility location and, where required, obtain organizational approval. | 6/19/15 |
| 9 | Deleted2a and 5a and revised 17 from castings to forgings Changed KomTek Technologies to KomTek Forge | 6/30/16 |
| 10 | Deleted 15e Parker end use | 1/4/17 |
| 11 | Deleted 3 All material furnished or processed under this order must be approved by Komtek Forge and/or a customer approved supplier list. | 6/28/17 |
| 12 | #14 Added “prior to” | 11/27/17 |
| 13 | Added 2a All master heats and bar sizes must be segregated by individual bundles. Small individual bundles can be bundled together as long as each bundle is properly identified during shipment. | 9/24/18 |
| 14 | Deleted S-1002 from paragraph 15a | 1/19/21 |